NE 795-014 Advanced Reactor Materials and Materials Performance

Exam 1

Please provide your responses to the following questions. Point values indicated expected depth of response.

- What are some key differences between high-temperature gas reactors and light water reactors? (8 pts)

- What are the individual layers in a TRISO particle? What purpose does each layer have? (14 pts)

- What are the trade-offs between carbide and oxide fuel kernels? (10 pts)

- How does the nature of irradiation damage accumulation change with irradiation temperature in SiC? (12 pts)

- How does thermal conductivity change in SiC with irradiation? What are the primary phonon scatterers, and why can someone determine the thermal conductivity from the swelling in SiC? (10 pts)

- Discuss the role of fission products on the failure of TRISO particles? (10 pts)

- How does graphite dimensionally change under irradiation? Why does this behavior happen? (14 pts)

- List three types of failure mechanisms for TRISO particles. (8 pts)

- Provide one example of an advanced TRISO concept and explain why it is of interest. (8 pts)

- What are three phenomena/behaviors that need to be accounted for in fuel performance modeling of TRISO fuels? What is one data need for fuel performance modeling? (6 pts)