Exam 1: NE591-10: Nuclear Fuel Performance

Show all work. Label question number in your response. You do not need to restate question.

- (12 pts) Derive the relationship for temperature drop over the fuel in cartesian coordinates assuming the following boundary conditions: T’(x0) = 0; x0 = 0; x1 = X; T(x1) = T1; and the simplified equation below:

∂∂xk∂T∂x+Q=0

What assumptions were made to get to the provided equation?

- (18 pts) The below fuel pellet has an exterior cladding coating. Calculate the centerline temperature, and the temperature at r=0.4 cm.

Outer coating k: 5 W/m-K; Cladding k: 15 W/m-K; Fuel k: 0.5 W/cm-K; Gap k: 25 W/m-K; Coolant h: 5.5 W/cm2-K; T(cool): 800 K; Q = 400 W/cm3;

r1=0.6 cm; r2= 0.8 cm; r3 = 0.85 cm; r4 = 0.86 cm;

- (14 pts) U3Si2 is a uranium silicide fuel being considered for use in light water reactors. Consider this fuel to have a thermal conductivity of 14.5 W/(m-K), an enrichment of 19.5%, and a density of 15.67 g/cm3. Assume the fission cross section is 570 barns.

  - What is the heat generation rate, given a neutron flux of 2E12 n/cm2-s?

  - What enrichment of UO2 would be required to obtain the same heat generation rate? UO2 density is 10.97 g/cc.

- (12 pts) Given a rod of 3 m in length, a LHR0 = 150 W/cm, and a  of 1.1:

  - What is the LHR at z=1.8 m?

  - Which coolant sees a larger change in outlet to inlet temperature?

    - water: CP = 4200 J/kg-K, mdot = 0.22 kg/s-rod

    - sodium: CP=1404 J/kg-K, mdot = 0.12 kg/s/rod

- (16 pts) Perform both a forward and a backward Euler time stepping to approximate the function. Compute with a timestep of dt=0.33, expanding to tn=2.

y(t) = 2t+3t3

t0 = 1

What is the amount of error from each approach.

- (5 pts) Explain the difference between fertile, fissile and fissionable.

- (4 pts) List two reasons why we don’t use pure metallic U as a fuel form?

- (4 pts) What is smear density? Why is this necessary?

- (8 pts) Why do we need to enrich U? What compound is utilized in the enrichment process? Describe the centrifuge-based enrichment of U, including why it works.

- (4 pts) Name two primary fission product species. Provide justification.

- (8 pts) What are the three ways that space is discretized for numerical solutions? Which type is used in state-of-the-art fuel performance simulations? Why?