Exam 2: NE591-10: Nuclear Fuel Performance

- For each of the following scenarios, does thermal expansion lead to stress? (6 pts)

a)

b)

c)

- Name three US fuel performance codes: (6 pts)

- What is the valence state of U in UO2? What are the possible valence states of U? (4 pts)

- Provide an example of a 0-D defect. Provide an example of a 3-D defect. (6 pts)

- Name three properties that vary as a function of stoichiometry in UO2. (6 pts)

- How does grain size affect the mechanical properties of a material? (8 pts)

- Define strain hardening. What causes strain hardening? (8 pts)

- Given the below stress/strain curve, answer the following questions:

  - Label the yield stress, the ultimate tensile stress, and the fracture stress. (6 pts)

  - How would one determine the Young’s modulus from the stress-strain curve? (4 pts)

- Describe the differences between elastic and plastic deformation. (8 pts)

- Consider a fuel rod with a pellet radius of 4.5 mm that is experiencing a linear heat rate of 250 W/cm. What is the maximum stress experienced by the pellet, assuming that the fuel has k = 0.1 W/cm-K, E = 290 GPa, ν = 0.3, and α = 8.2e-6 1/K? (12 pts)

- Consider the stress state in a zircaloy fuel rod pressurized to 20 MPa with an average

radius of 5.6 mm and a cladding thickness of 0.6 mm.

- Calculate all three components of the stress using the thin walled cylinder approximation. (6 pts)

- Calculate all three components of the stress at the midpoint assuming a thick-walled cylinder. (8 pts)

- Calculate the maximum strain, with the stress components from (b) and with E=180 GPa and =0.28. (4 pts)

- Calculate the centerline temperature of the fuel before and after thermal expansion. Rf = 0.5 cm. tgap = 0.02 cm, TCO = 450 K, kfuel = 0.05 W/cm-K, kgap = 0.04 W/cm-K, kclad = 0.15 W/cm-K, LHR = 325 W/cm, c = 4.5x10-6 1/K, f = 15x10-6 1/K, Tref(fuel=clad) = 300 K. (18 pts).

Remember: